# Introduction

Human error is still one of the most frequent causes of catastrophes and ecological disasters. The main reason is that the monitoring systems concern only the state of the processes whereas human contribution to the overall performance of the system is left unsupervised. Since the control instruments are automated to a large extent, a human – operator becomes a passive observer of the supervised system, which results in weariness and vigilance drop. Thus, he may not notice important changes of indications causing financial or ecological consequences and a threat to human life. It therefore is crucial to assure that the operator’s conscious brain is involved in an active system supervising over the whole work time period.

It is possible to measure indirectly the level of the operator’s conscious brain involvement using eye motility analysis. Although there are capable sensors available on the market, a complex solution enabling transformation, analysis and reasoning based on measured signals still does not exist. In large control rooms, wiring the operator to the central system is a serious limitation of his mobility and disables his operation. Utilization of wireless technology becomes essential.

BlueEyes - the system developed by our team is intended to be the complex solution for monitoring and recording the operator’s conscious brain involvement as well as his physiological condition. This required designing a Personal Area Network linking all the operators and the supervising system. As the operator using his sight and hearing senses the state of the controlled system, the supervising system will look after his physiological condition.

# System overview

BlueEyes system provides technical means for monitoring and recording the operator’s basic physiological parameters. The most important parameter is saccadic activity, which enables the system to monitor the status of the operator’s visual attention along with head acceleration, which accompanies large displacement of the visual axis (saccades larger than 15 degrees). Complex industrial environment can create a danger of exposing the operator to toxic substances, which can affect his cardiac, circulatory and pulmonary systems. Thus, on the grounds of plethysmographic signal taken from the forehead skin surface, the system computes heart beat rate and blood oxygenation.

The BlueEyes system checks above parameters against abnormal (e.g. a low level of blood oxygenation or a high pulse rate) or undesirable (e.g. a longer period of lowered visual attention) values and triggers user-defined alarms when necessary.

Quite often in an emergency situation operators speak to themselves expressing their surprise or stating verbally the problem. Therefore, the operator’s voice, physiological parameters and an overall view of the operating room are recorded. This helps to reconstruct the course of operators’ work and provides data for long-term analysis.

Our system consists of a mobile measuring device and a central analytical system. The mobile device is integrated with Bluetooth module providing wireless interface between sensors worn by the operator and the central unit. ID cards assigned to each of the operators and adequate user profiles on the central unit side provide necessary data personalization so different people can use a single mobile device (called hereafter DAU – Data Acquisition Unit)

Figure 1. Overall system diagram

. The overall system diagram is shown in Figure 1.

The tasks of the mobile Data Acquisition Unit are to maintain Bluetooth connections, to get information from the sensor and sending it over the wireless connection, to deliver the alarm messages sent from the Central System Unit to the operator and handle personalized ID cards. Central System Unit maintains the other side of the Bluetooth connection, buffers incoming sensor data, performs on-line data analysis, records the conclusions for further exploration and provides visualization interface.

## Performance requirements

The portable nature of the mobile unit results in a number of performance requirements. As the device is intended to run on batteries, low power consumption is the most important constraint. Moreover, it is necessary to assure proper timing while receiving and transmitting sensor signals. To make the operation comfortable the device should be lightweight and electrically safe. Finally, the use of standard and inexpensive ICs will keep the price of the device at relatively low level.

The priority of the central unit is to provide real-time buffering of incoming sensor signals and semi-real-time processing of the data, which requires speed-optimized filtering and reasoning algorithms. Moreover, the design should assure the possibility of distributing the processing among 2 or more central unit nodes (e.g. to offload the database system related tasks to a dedicated server).

## Design methodologies

In creating the BlueEyes system a waterfall software development model was used since it is suitable for unrepeatable and explorative projects. During the course of the development UML standard notations were used. They facilitate communication between team members; all the ideas are clearly expressed by means of various diagrams, which is a sound base for further development.

The results of the functional design phase were documented on use case diagrams (Fig. 2, 3 and 4). During the low-level design stage the whole system was divided into five main modules (Fig. 1 – thick frames). Each of them has an independent, well-defined functional interface providing precise description of the services offered to the other modules. All the interfaces are documented on UML class-, interaction- and state diagrams. At this point each of the modules can be assigned to a team member, implemented and tested in parallel. The last stage of the project is the integrated system testing.

## Innovative ideas

The unique feature of our system relies on the possibility of monitoring the operator’s higher brain functions involved in the acquisition of the information from the visual environment. The wireless link between the sensors worn by the operator and the supervising system offers new approach to system overall reliability and safety. This gives a possibility to design a supervising module whose role is to assure the proper quality of the system performance. The new possibilities can cover such areas as industry, transportation (by air, by road and by sea), military command centers or operating theaters (anesthesiologists).

# Implementation and engineering considerations

  - Functional design

Figure 2. Mobile device use cases

During the functional design phase we used UML standard use case notation, which shows the functions the system offers to particular users. We defined three groups of users: operators, supervisors and system administrators.

Operator is a person whose physiological parameters are supervised. The operator wears the DAU. The only functions offered to that user are authorization in the system and receiving alarm alerts. Such limited functionality assures the device does not disturb the work of the operator (Fig. 2).

Authorization – the function is used when the operator’s duty starts. After inserting his personal ID card into the mobile device and entering proper PIN code the device will start listening for incoming Bluetooth connections. Once the connection has been established and authorization process has succeeded (the PIN code is correct) central system starts monitoring the operator’s physiological parameters. The authorization process shall be repeated after reinserting the ID card. It is not, however, required on reestablishing Bluetooth connection.

Receiving alerts – the function supplies the operator with the information about the most important alerts regarding his or his co-workers’ condition and mobile device state (e.g. connection lost, battery low). Alarms are signaled by using a beeper, earphone providing central system sound feedback and a small alphanumeric LCD display, which shows more detailed information.

Figure 3. Functionality offered to the supervisor

Supervisor is a person responsible for analyzing operators’ condition and performance. The supervisor receives tools for inspecting present values of the parameters (On-line browsing) as well as browsing the results of long-term analysis (Off-line browsing).

During the on-line browsing it is possible to watch a list of currently working operators and the status of their mobile devices. Selecting one of the operators enables the supervisor to check the operator’s current physiological condition (e.g. a pie chart showing active brain involvement) and a history of alarms regarding the operator. All new incoming alerts are displayed immediately so that the supervisor is able to react fast.

However, the presence of the human supervisor is not necessary since the system is equipped with reasoning algorithms and can trigger user-defined actions (e.g. to inform the operator’s co-workers).

During off-line browsing it is possible to reconstruct the course of the operator’s duty with all the physiological parameters, audio and video data. A comprehensive data analysis can be performed enabling the supervisor to draw conclusions on operator’s overall performance and competency (e.g. for working night shifts).

Figure 4. Administrative functionality

System administrator is a user that maintains the system. The administrator is delivered tools for adding new operators to the database, defining alarm conditions, configuring logging tools and creating new analyzer modules.

While registering new operators the administrator enters appropriate data (and a photo if available) to the system database and programs his personal ID card.

Defining alarm conditions – the function enables setting up user-defined alarm conditions by writing condition-action rules (e.g. low saccadic activity during a longer period of time  inform operator’s co-workers, wake him up using the beeper or playing appropriate sound and log the event in the database).

Designing new analyzer modules – based on earlier recorded data the administrator can create new analyzer modules that can recognize other behaviors than those which are built-in the system. The new modules are created using decision tree induction algorithm. The administrator names the new behavior to be recognized and points the data associated with it. The results received from the new modules can be used in alarm conditions.

Monitoring setup enables the administrator to choose the parameters to monitor as well as the algorithms of the desired accuracy to compute parameter values.

Logger setup provides tools for selecting the parameters to be recorded. For audio data sampling frequency can be chosen. As regards the video signal, a delay between storing consecutive frames can be set (e.g. one picture every two seconds).

Database maintenance – here the administrator can remove old or “uninteresting” data from the database. The “uninteresting” data is suggested by the built-in reasoning system.

  - Data Acquisition Unit (DAU)

In this section we describe the hardware part of the BlueEyes system with regard to the physiological data sensor, the DAU hardware components and microcontroller software.

### Physiological data sensor

To provide the Data Acquisition Unit with necessary physiological data we decided to purchase an off-shelf eye movement sensor – Jazz Multisensor. It supplies raw digital data regarding eye position, the level of blood oxygenation, acceleration along horizontal and vertical axes and ambient light intensity.

Figure 5. Jazz Multisensor

Eye movement is measured using direct infrared oculographic transducers.

The eye movement is sampled at 1kHz, the other parameters at 250 Hz. The sensor sends approximately 5,2kB of data per second.

### Hardware specification

We have chosen Atmel 8952 microcontroller to be the core of the Data Acquisition Unit since it is a well-established industrial standard and provides necessary functionality (i.e. high speed serial port) at a low price. The figure below shows the other DAU components (for detailed DAU schematics – see Appendix A).

Figure 6. DAU hardware diagram

Since the Bluetooth module we received supports synchronous voice data transmission (SCO link) we decided to use hardware PCM codec to transmit operator’s voice and central system sound feedback. The codec that we have employed reduces the microcontroller’s tasks and lessens the amount of data being sent over the UART. Additionally, the Bluetooth module performs voice data compression, which results in smaller bandwidth utilization and better sound quality. We have chosen Motorola MC145483 linear 13-bit 3.3V codec to be used in the prototype, since it is voltage-level compatible with the Bluetooth module.

Communication between the Bluetooth module and the microcontroller is carried on using standard UART interface. MAX232 Level Shifter does the RS232  TTL voltage level conversion. The speed of the UART is set to 115200 bps in order to assure that the entire sensor data is delivered in time to the central system (sensor data along with Bluetooth-related control data take up approx. 70% of the bandwidth).

The alphanumeric LCD display (two 8-character lines) gives more information of incoming events and helps the operator enter PIN code. In the prototype we used inexpensive Hitachi command-set LCD display.

The LED indicators show the results of built-in self-test, power level and the state of wireless connection.

The simple keyboard is used to react to incoming events (e.g. to silence the alarm sound) and to enter PIN code while performing authorization procedure.

ID card interface helps connect the operator’s personal identification card to the DAU. After inserting the card authorization procedure starts. In the prototype as the ID card an I2C EEPROM (24C16) was used since it provides easy programming interface and is capable of holding all the necessary operator’s data. In the commercial release a cryptographic processor should be used instead. Each ID card is programmed to contain: operator’s unique identifier, device access PIN code the operator enters on inserting his ID card and system access PIN code that is used on connection authentication. The operator’s unique identifier enables the supervising system to distinguish different operators.

### Microcontroller software specification

All the DAU software is written in 8051 assembler code, which assures the highest program efficiency and the lowest resource utilization (for detailed microcontroller software description – see Appendix B). During the development we used GNU licensed 8051 assembler compiler – as31.

At a low-level design stage we modeled the software using a state diagram. Such a diagram facilitates implementation, debugging and testing phases.

Figure 7. DAU state diagram

In each state the DAU communicates with the Bluetooth module using Host Controller Interface (HCI) commands.

In the No ID card state a self-test is performed to check if the device is working correctly. After the self-test passes the sensor and Bluetooth module are reset and some initialization commands are issued (i.e. HCI_Reset, HCI_Ericsson_Set_UART_Baud_Rate etc.). Once the initialization has been successfully completed the device starts to check periodically for ID card presence by attempting to perform an I2C start condition. When the attempt succeeds and the operator’s identifier is read correctly the device enters User authorization state.

In the User authorization state the operator is prompted to enter his secret PIN code. If the code matches the code read from the inserted ID card the device proceeds waiting for incoming Bluetooth connections.

On entering Waiting for connection state the DAU puts the Bluetooth module in Inquiry and Page Scan mode. After the first connection request appears, the DAU accepts it and enters Connection authentication state.

In the Connection authentication state the DAU issues Authentication_Requested HCI command. On Link Controller’s Link_Key_Request the DAU sends Link_Key_Negative_Reply in order to force the Bluetooth module to generate the link key based on the supplied system access PIN code. After a successful authentication the DAU enters the Data Processing state, otherwise it terminates the connection and enters the Waiting for connection state.

Figure 8. Jazz sensor frame format

The main DAU operation takes place in the Data Processing state. In the state five main kinds of events are handled. Since the sensor data has to be delivered on time to the central system, data fetching is performed in high-priority interrupt handler procedure. Every 4ms the Jazz sensor raises the interrupt signaling the data is ready for reading. The following data frame is used:

Figure 9. Jazz Sensor data fetching waveforms

The preamble is used to synchronize the beginning of the frame, EyeX represents the horizontal position of the eye, EyeY – vertical, AccX and AccY – the acceleration vectors along X and Y axes, PulsoOxy, Batt and Light – blood oxygenation, voltage level and light intensity respectively. The figure below shows the sensor communication timing.

The received data is stored in an internal buffer; after the whole frame is completed the DAU encapsulates the data in an ACL frame and sends it over the Bluetooth link. The fetching phase takes up approx. 192s (24 frames x 8s) and the sending phase takes at 115200 bps approx. 2,8 ms, so the timing fits well in the 4ms window. Omitting the acknowledgement signal allows faster transmission.

The second group of events handled in the Data Processing state are system messages and alerts. They are sent from the central system using the Bluetooth link. Since the communication also uses microcontroller’s interrupt system the events are delivered instantly. More detailed considerations regarding handling of the Bluetooth- and sensor-related interrupts are presented in section 3.2.4 Tradeoffs and optimizations.

The remaining time of the microcontroller is utilized performing LCD display, checking the state of the buttons, ID card presence and battery voltage level. Depending on which button is pressed appropriate actions are launched (e.g. dismissing alarm sound). In case the voltage level is low the operator and the central system are notified.

In every state removing the ID card causes the device to enter the No ID card state, terminating all the established connections.

### DAU Project and implementation tradeoffs and optimizations

In the DAU there are two independent data sources – the Jazz sensor and Bluetooth Host Controller. Since they are both handled using the interrupt system it is necessary to decide which of the sources should have higher priority. Giving the sensor data the highest priority may result in losing some of the data sent by the Bluetooth module, as the transmission of the sensor data (192 s) takes twice as much time as receiving one byte from UART (87 s). Missing one single byte sent from the Bluetooth causes the loss of control over the transmission. On the other hand, giving the Bluetooth the highest priority will make the DAU stop receiving the sensor data until the Host Controller finishes its transmission. In this case the interrupted sensor frame shall be discarded. We do not consider the data being sent from the DAU to the Bluetooth as it does not affect the operation. Since missing one byte of the Bluetooth communication affects functioning of the DAU much more than losing one single sensor data frame (which can be estimated at the central system) we have chosen the second option. Central system alerts are the only signals that can appear during sensor data fetching after all the unimportant Bluetooth events have been masked out. The best solution would be to make the central unit synchronize the alerts to be sent with the Bluetooth data reception. As the delivered operating system is not a real-time system, the full synchronization is not possible. We therefore have calculated the probability of interrupting the sensor data transmission by a single alert.

The interruption may occur when there is a collision between sensor data fetching and an asynchronously incoming central system alert. The situation is shown in the figure below.

Figure 10. Collision condition timing

The alert message is approx. 20 bytes long (including HCI headers), thus its reception time is approx. 1.6 ms. The probability of the collision Pc equals Tc/Tp = 1.792ms / 4ms = 0.44, which means every second alert request discards one sensor data frame. Assuming that during normal system operation the alert message arrivals are less frequent than one per 15 minutes – only one of more than 200,000 sensor data frames is lost. Moreover, while buffering at the central system the missing frame is reconstructed based on the previous and next sensor data packet (each frame’s preamble contains a sequential number and a checksum).

As the Bluetooth module communicates asynchronously with the microcontroller there was a need of implementing a cyclic serial port buffer, featuring UART CTS/RTS flow control and a producer-consumer synchronization mechanism.

At the low-level design stage we calculated the amount of data to be sent to the Bluetooth device over UART interface. The sensor data along with Bluetooth control command occupy approx. 70% of the available 115200 bps bandwidth. Thus, if we wanted to send the operator’s voice data over the same channel this would dramatically increase microcontroller’s load and affect sound quality. Introducing hardware PCM codec and using the Bluetooth synchronous connection (SCO link) eliminates those constraints.

During the DAU hardware design and implementation we put much emphasis on optimizing price and power consumption of the device. To achieve this we used standard low-voltage ICs where possible. Moreover, the microcontroller software enables power-down modes of the ICs when it does not disturb the function of the device (i.e. when no ID card is inserted the PCM codec and the Bluetooth module are put in low-power mode). All those optimizations help keep the time between recharging the batteries long.

### DAU Validation and testing

While testing the DAU software we used two techniques. At the implementation stage we were performing a white-box testing. After completing subsequent portions of an assembler code its creator checked the program for hidden or logical errors. Then, another team member checked the results. After the implementation phase finished, based on the state diagram we performed a black-box testing. All the transitions between the states have been checked against possible implementation errors. This assured us that every state is reached only under the conditions specified on the diagram. After the implementation of the central system finished we performed integrated system tests.

  - Central System Unit (CSU)

CSU software is located on the delivered Toshiba laptop, in case of larger resource demands the processing can be distributed among a number of nodes. In this section we describe the four main CSU modules (see Fig. 1): Connection Manager, Data Analysis, Data Logger and Visualization. The modules exchange data using specially designed single-producer-multi-consumer buffered thread-safe queues. Any number of consumer modules can register to receive the data supplied by a producer. Every single consumer can register at any number of producers, receiving therefore different types of data. Naturally, every consumer may be a producer for other consumers (for system core class diagram – see Appendix C). This approach enables high system scalability – new data processing modules (i.e. filters, data analyzers and loggers) can be easily added by simply registering as a consumer.

### Connection Manager

Figure 11. Connection Manager components

Connection Manager’s main task is to perform low-level Bluetooth communication using Host Controller Interface commands. It is designed to cooperate with all available Bluetooth devices in order to support roaming (for CSU detailed hardware specification – see Appendix D). Additionally, Connection Manager authorizes operators, manages their sessions, demultiplexes and buffers raw physiological data. Figure 11 shows Connection Manager architecture.

Transport Layer Manager hides the details regarding actual Bluetooth physical transport interface (which can be either RS232 or UART or USB standard) and provides uniform HCI command interface.

Bluetooth Connection Manager is responsible for establishing and maintaining connections using all available Bluetooth devices. It periodically inquires new devices in an operating range and checks whether they are registered in the system database. Only with those devices the Connection Manager will communicate. After establishing a connection an authentication procedure occurs. The authentication process is performed using system PIN code fetched from the database. Once the connection has been authenticated the mobile unit sends a data frame containing the operator’s identifier. Finally, the Connection Manager adds a SCO link (voice connection) and runs a new dedicated Operator Manager, which will manage the new operator’s session (for detailed Bluetooth communication flow charts and protocols – see Appendix E). Additionally, the Connection Manager maps the operator’s identifiers into the Bluetooth connections, so that when the operators roam around the covered area a connection with an appropriate Bluetooth device is established and the data stream is redirected accordingly.

The data of each supervised operator is buffered separately in the dedicated Operator Manager. At the startup it communicates with the Operator Data Manager in order to get more detailed personal data. The most important Operator Manager’s task is to buffer the incoming raw data and to split it into separate data streams related to each of the measured parameters. The raw data is sent to a Logger Module, the split data streams are available for the other system modules through producer-consumer queues. Furthermore, the Operator Manager provides an interface for sending alert messages to the related operator.

Operator Data Manager provides an interface to the operator database enabling the other modules to read or write personal data and system access information.

### Data Analysis Module

The module performs the analysis of the raw sensor data in order to obtain information about the operator’s physiological condition. The separately running Data Analysis Module supervises each of the working operators. The module consists of a number of smaller analyzers extracting different types of information. Each of the analyzers registers at the appropriate Operator Manager or another analyzer as a data consumer and, acting as a producer, provides the results of the analysis. An analyzer can be either a simple signal filter (e.g. Finite Input Response (FIR) filter) or a generic data extractor (e.g. signal variance, saccade detector) or a custom detector module. As we are not able to predict all the supervisors’ needs, the custom modules are created by applying a supervised machine learning algorithm to a set of earlier recorded examples containing the characteristic features to be recognized. In the prototype we used an improved C4.5 decision tree induction algorithm. The computed features can be e.g. the operator’s position (standing, walking and lying) or whether his eyes are closed or opened.

As built-in analyzer modules we implemented a saccade detector, visual attention level, blood oxygenation and pulse rate analyzers.

The saccade detector registers as an eye movement and accelerometer signal variance data consumer and uses the data to signal saccade occurrence. Since saccades are the fastest eye movements the algorithm calculates eye movement velocity and checks physiological constraints.

The algorithm we used performs two main steps:

A. User adjustment step. The phase takes up 5 s. After buffering approx. 5 s of the signal differentiate it using three point central difference algorithm, which will give eye velocity time series. Sort the velocities by absolute value and calculate upper 15% of the border velocity along both X – v0x and Y – v0y axes . As a result v0x and v0y are cut-off velocities.

B. On-line analyzer flow. Continuously calculate eye movement velocity using three point central difference algorithm. If the velocity excess precalculated v0 (both axes are considered separately) there is a possibility of saccade occurrence. Check the following conditions (if any of them is satisfied do not detect a saccade):

- the last saccade detection was less than 130 ms ago (physiological constraint – the saccades will not occur more frequently)

- the movement is nonlinear (physiological constraint)

- compare the signal with accelerometer (rapid head movement may force eye activity of comparable speed)

- if the accelerometer signal is enormously uneven consider ignoring the signal due to possible sensor device movements.

- If none of the above conditions is satisfied – signal the saccade occurrence.

The visual attention level analyzer uses as input the results produced by the saccade detector. Low saccadic activity (large delays between subsequent saccades) suggests lowered visual attention level (e.g. caused by thoughtfulness). Thus, we propose a simple algorithm that calculates the visual attention level (Lva): Lva = 100/ts10, where ts10 denotes the time (in seconds) occupied by the last ten saccades. Scientific research has proven [1] that during normal visual information intake the time between consecutive saccades should vary from 180 up to 350 ms. This gives Lva at 28 up to 58 units. The values of Lva lower than 25 for a longer period of time should cause a warning condition. The following figure shows the situation where the visual attention lowers for a few seconds.

saccade

Figure 12. Saccade occurrences and visual attention level

The Pulse rate analyzer registers for the oxyhemoglobin and deoxyhemoglobin level data streams. Since both signals contain a strong sinusoidal component related to heartbeat, the pulse rate can be calculated measuring the time delay between subsequent extremes of one of the signals. We decided not to process only one of the data streams – the algorithm is designed to choose dynamically one of them on the grounds of the signal level. Unfortunately, the both signals are noised so they must be filtered before further processing. We considered a number of different algorithms and decided to implement average value based smoothing. More detailed discussion is presented in section 3.3.5 Tradeoffs and Optimization. The algorithm consists in calculating an average signal value in a window of 100 samples. In every step the window is advanced 5 samples in order to reduce CPU load. This approach lowers the sampling rate from 250 Hz down to 50 Hz. However, since the heartbeat frequency is at most 4 Hz the Nyquist condition remains satisfied. The figures show the signal before (Fig. 13a) and after filtering (Fig 13b).

Figure 13b.

maximum

minimum

Figure 13a.

After filtering the signal the pulse calculation algorithm is applied. The algorithm chooses the point to be the next maximum if it satisfies three conditions: points on the left and on the right have lower values, the previous extreme was a minimum, and the time between the maximums is not too short (physiological constraint). The new pulse value is calculated based on the distance between the new and the previous maximum detected. The algorithm gets the last 5 calculated pulse values and discards 2 extreme values to average the rest. Finally, it does the same with the minimums of the signal to obtain the second pulse rate value, which gives the final result after averaging.

Additionally, we implemented a simple module that calculates average blood oxygenation level. Despite its simplicity the parameter is an important measure of the operator’s physiological condition.

The other signal features that are not recognized by the built-in analyzers can be extracted using custom modules created by Decision Tree Induction module. The custom module processes the generated decision tree, registers for needed data streams and produces the desired output signal.

Decision Tree Induction module generates the decision trees, which are binary trees with an attribute test in each node. The decision tree input data is an object described by means of a set of attribute-value pairs. The algorithm is not able to process time series directly. The attributes therefore are average signal value, signal variance and the strongest sinusoidal components. As an output the decision tree returns the category the object belongs to.

In the Decision Tree Induction module we mainly use C4.5 algorithm [2], but also propose our own modifications (for details regarding the C4.5 algorithm – see Appendix F). The algorithm is a supervised learning from examples i.e. it considers both attributes that describe the case and a correct answer. The main idea is to use a divide-and-conquer approach to split the initial set of examples into subsets using a simple rule (i-th attribute less than a value). Each division is based on entropy calculation – the distribution with the lowest entropy is chosen. Additionally, we propose many modifications concerning some steps of the algorithm and further exploration of the system.

| Algorithm step | Original C4.5 | Proposed modification |
| --- | --- | --- |
| Entropy calculation | E(S)=−j=1kf(Ci,S)S*log2f(Ci,S)S where f(x,y)denotes the frequency of decision class x in set y | Decision not based on entropy. “Common sense” rules for splitting input set. |
| Choosing best division | Minimization of weighted entropy sum of subsets | Additional factor that markedly favors balanced divisions |
| Division condition | Condition attribute ai < v | Additional condition attribute ai∈a,b |
| Tree pruning | Prune the tree to maximize the test set hit ratio | Stop splitting input set when it is too small (much faster than the other method). No tree modifications after testing. |

For each case to be classified C4.5 traverses the tree until reaching the leaf where appropriate category id is stored. To increase the hit ratio our system uses more advanced procedure. For single analysis we develop a group of k trees (where k is a parameter), which we call a decision forest. Initial example set S is divided randomly into k+1 subsets S0,...,Sk. S0 is left to test the whole decision forest. Each tree is induced using various modifications of the algorithm to provide results’ independence. Each i-th tree is taught using S\S0\Si set (S without S0 and Si sets) and tested with Si that estimates a single tree error ratio. Furthermore we extract all wrongly classified examples and calculate correlation matrix between each pair of the trees. In an exploring phase we use unequal voting rule – each tree has a vote of strength of its reliability. Additionally, if two trees give the same answer their vote is weakened by the correlation ratio.

Alarm Dispatcher Module is a very important part of the Data Analysis module. It registers for the results of the data analysis, checks them with regard to the user-defined alarm conditions and launches appropriate actions when needed. The module is a producer of the alarm messages, so that they are accessible in the logger and visualization modules.

### Data Logger Module

The module provides support for storing the monitored data in order to enable the supervisor to reconstruct and analyze the course of the operator’s duty. The module registers as a consumer of the data to be stored in the database (for database schema – see Appendix G). Each working operator’s data is recorded by a separate instance of the Data Logger. Apart from the raw or processed physiological data, alerts and operator’s voice are stored. The raw data is supplied by the related Operator Manager module (Fig. 11), whereas the Data Analysis module delivers the processed data. The voice data is delivered by a Voice Data Acquisition module. The module registers as an operator’s voice data consumer and optionally processes the sound to be stored (i.e. reduces noise or removes the fragments when the operator does not speak). The Logger’s task is to add appropriate time stamps to enable the system to reconstruct the voice.

Additionally, there is a dedicated video data logger, which records the data supplied by the Video Data Acquisition module (in the prototype we use JPEG compression). The module is designed to handle one or more cameras using Video For Windows standard. The cameras can be either directly connected to the system or accessible through a UDP network connection (for detailed camera protocol specification – see Appendix H). The Data Logger is able to use any ODBC-compliant database system. In the prototype we used MS SQL Server, which is a part of the Project Kit.

### Visualization Module

The module provides user interface for the supervisors. It enables them to watch each of the working operator’s physiological condition along with a preview of selected video source and his related sound stream. All the incoming alarm messages are instantly signaled to the supervisor. Moreover, the visualization module can be set in the off-line mode, where all the data is fetched from the database. Watching all the recorded physiological parameters, alarms, video and audio data the supervisor is able to reconstruct the course of the selected operator’s duty (for detailed Visualization Module structure – see Appendix I).

### CSU Project and implementation tradeoffs and optimizations

The prototype we have built has several limitations, which are not the result of the project deficiency but are rather caused by the constraints imposed by the Project Kit and small budget. In the commercial release the USB web-cam should be replaced by an industrial camera, connected to a capturing device (optionally a video signal multiplexer could be used). The use of such a camera would lessen CPU load and improve the video signal quality. Since the Bluetooth module we received does not support redirecting of the voice SCO connections’ data to the serial port (which is a part of the Bluetooth specification) we had to build a PCM interface on the central system side similar to the one used in the DAU. This makes the Voice Data Acquisition module receive the sound using the sound card.

Implementing the pulse rate analyzer we considered several filtering algorithms. We tested FIR (Finite Impulse Response) digital filters, whose output signal is easier to analyze. However, the filter needs ten times as many samples as our algorithm to produce satisfactory results. It makes the FIR filter consume much more CPU processing power. The use of FFT algorithm is pointless, as it needs over 20 000 samples to achieve the desired accuracy (i.e. 0.01 Hz).

While designing and implementing the reasoning algorithm, apart from the decision trees, we considered neural networks and k-nn (k-nearest neighbors) algorithm. We have chosen the decision trees, as the other algorithms are not able to explain their decisions. Traversing the tree nodes it is possible to identify all the perquisites of the decision, whereas neural network and k-nn coefficients do not have clear interpretation. Additionally, based on the decision tree structure the system is able to generate equivalent decision rule set.

### CSU Validation and testing

While implementing and testing particular central system modules we used about a hundred disk files containing prerecorded sensor data. The data was specially selected to contain the behaviors we were interested in (e.g. saccades, breathing, low and high pulse rate, standing, walking, lying). The files were prepared using a dedicated tool (section 3.4). As all the central system modules communicate using producer-consumer queues it was possible to implement and test the modules independently. In order to achieve it we created temporary producers serving the data read from the selected file. The next testing step was to create a producer reading the data directly from the sensor using a parallel port. This enabled us to test the Data Analysis, Logger and Visualization modules working in a cooperation with a real-time data source. At this stage we generated a custom analyzer module which determines the position of the operator (standing, lying, walking). After testing we found out that the generated module properly classifies the position (for the details regarding the decision trees – see Appendix J). We also did some minor algorithm and database access optimizations. In the integrated system testing phase we added the wireless communication facility activating the DAU and Connection Manager modules. The last task was to check the Bluetooth communication performance with regard to the number of lost data frames.

  - Tools developed during the course of the project

In creating the hardware part of the DAU we built a development board, which enabled us to mount, connect and test various peripheral devices cooperating with the microcontroller.

During the implementation of the DAU we needed a piece of software to establish and test Bluetooth connections. We therefore created a tool called BlueDentist (Fig. 14). The tool provides support for controlling the currently connected Bluetooth device. Its functions are: local device management (resetting, reading local BD_ADDR, putting in Inquiry/Page and Inquiry/Page scan modes, reading the list of locally supported features and setting UART speed) and connection management (receiving and displaying Inquiry scan results, establishing ACL links, adding SCO connections, performing link authorization procedure, sending test data packets and disconnecting).

To test the possibilities and performance of the remaining parts of the Project Kit (computer, camera and database software) we created BlueCapture (Fig. 15). The tool supports capturing video data from various sources (USB web-cam, industrial camera) and storing the data in the MS SQL Server database. Additionally, the application performs sound recording. After filtering and removing insignificant fragments (i.e. silence) the audio data is stored in the database. Finally, the program plays the recorded audiovisual stream. We used the software to measure database system performance and to optimize some of the SQL queries (e.g. we replaced correlated SQL queries with cursor operations). Since all the components of the application have been tested thoroughly they were reused in the final software, which additionally reduced testing time.

We also created a simple tool for recording Jazz Multisensor measurements. The program reads the data using a parallel port and writes it to a file.

To program the operator’s personal ID card we use a standard parallel port, as the EEPROMs and the port are both TTL-compliant. A simple dialog-based application helps to accomplish the task (for detailed ID card system specification see – Appendix K).

Figure 14. BlueDentist

Figure 15. BlueCapture

# Summary

We have decided to develop the BlueEyes system because of the need for a real-time monitoring system for a human operator. The approach is innovative since it helps supervise the operator not the process, as it is in presently available solutions. We hope the system in its commercial release will help avoid potential threats resulting from human errors, such as weariness, oversight, tiredness or temporal indisposition. However, the prototype we have developed is a good estimation of the possibilities of the final product.

We find it possible still to improve our project. The use of a miniature CMOS camera integrated into the eye movement sensor will enable the system to calculate the point of gaze and observe what the operator is actually looking at. Introducing voice recognition algorithm will facilitate the communication between the operator and the central system and simplify authorization process.

Despite considering in the report only the operators working in control rooms, our solution may well be applied to everyday life situations. Assuming the operator is a driver and the supervised process is car driving it is possible to build a simpler embedded on-line system, which will only monitor conscious brain involvement and warn when necessary. As in this case the logging module is redundant, and the Bluetooth technology is becoming more and more popular, the commercial implementation of such a system would be relatively inexpensive.

The final thing is to explain the name of our system. BlueEyes emphasizes the foundations of the project – Bluetooth technology and the movements of the eyes. Bluetooth provides reliable wireless communication whereas the eye movements enable us to obtain a lot of interesting and important information.

# References

- Carpenter R. H. S., Movements of the eyes, 2nd edition, Pion Limited, 1988, London.

- Quinlan J. R., C4.5 programs for machine learning, Morgan Kaufmann Publishers, 1993, San Mateo, California.

- Horowitz P., Hill W., The art of Electronics, Cambridge University Press, 1989.

- Bluetooth specification, version 1.0B, Bluetooth SIG, 1999.

- ROK 101 007 Bluetooth Module, Ericsson Microelectronics, 2000.

- MC145483 3V 13-bit Linear PCM Codec-Filter, Motorola Semiconductor, 1997.

- AT89C52 8-bit Microcontroller Datasheet, Atmel.

- ST14C16 memory card IC Datasheet, ST Microelectronics, 1999.

- Leinecker R. C., Archer T., Visual C++ Bible, IDG Books, 1999.

- MSDN Library, Microsoft (Visual Studio 6.0).

- Intel Signal Processing Library – Reference Manual.

# Appendices

  - Appendix A. Data Acquisition Unit hardware schematics

### Microcontroller board

Components

- ICs

  - IC1 		 	Atmel 89c52 microcontroller

  - IC2 		 	MAX232

- Capacitors

  - C1, C2, C6, C7	10u/16V

  - C3, C4			33p

  - C5			10u/10V

- Resistors

  - R1, R2			1k

  - R3, R4			330

  - R5, R7			360

  - RN1			8x10k

  - R8			10k

- Transistors

  - T1, T2			BC237

- Crystal

  - Q1			22,1184 MHz

- Diodes

  - D1, D2		LED, round 3mm, blue

  - D3			1N4148

- Connectors

  - CONN1		power supply with voltage level monitor

  - CONN2		LCD display

  - CONN3		Bluetooth-serial port

  - CONN4		ID-card interface

  - CONN5		keyboard connector

  - CONN6		Jazz sensor interface

  - CONN7		Bluetooth hardware reset output

- Jumper

  - JP1			enable/disable power protection

- Other

  - SG1			buzzer with internal generator

Connectors

| Connector | PIN number | Signal |
| --- | --- | --- |
| CONN1 Power connector | 1 | Ground |
|  | 2 | Voltage control input |
|  | 3 | +5V |
| CONN2 LCD display connector | 1 | Ground |
|  | 2 | Contrast adjustment |
|  | 3 | RS |
|  | 4 | E |
|  | 5 | D3 |
|  | 6 | D2 |
|  | 7 | D1 |
|  | 8 | D0 |
|  | 9 | +5V |
| CONN3 Bluetooth serial port | 1 | Ground |
|  | 2 | RX (input) |
|  | 3 | RTS (input) |
|  | 4 | CTS (output) |
|  | 5 | TX (output) |
| CONN4 I2C EEPROM ID-card | 1 | Ground |
|  | 2 | SDA |
|  | 3 | SCL |
|  | 4 | +5V |
| CONN5 Keyboard connector | 1 | Ground |
|  | 2 | Left |
|  | 3 | Up |
|  | 4 | Enter |
|  | 5 | Down |
|  | 6 | Right |
| CONN6 Jazz sensor interface | 1 | +5V |
|  | 2 | D0 |
|  | 3 | D1 |
|  | 4 | D2 |
|  | 5 | D3 |
|  | 6 | D4 |
|  | 7 | D5 |
|  | 8 | D6 |
|  | 9 | D7 |
|  | 10 | /STROBE (input) |
|  | 11 | Reset (output) |
|  | 12 | Ground |
| CONN7 Bluetooth hardware reset | 1 | RESET |
|  | 2 | Ground |

Microcontroller port lines assignement

| Microcontroller port |  |  |  | Signal |
| --- | --- | --- | --- | --- |
| Port |  | Descr. | Pin no. |  |
| P0 | P0.0 | AD0 | 39 | Keyboard – Right |
|  | P0.1 | AD1 | 38 | Keyboard – Down |
|  | P0.2 | AD2 | 37 | Keyboard – Enter |
|  | P0.3 | AD3 | 36 | Keyboard – Up |
|  | P0.4 | AD4 | 35 | Keyboard – Left |
|  | P0.5 | AD5 | 34 | Buzzer |
|  | P0.6 | AD6 | 33 | I2C EEPROM – SCL |
|  | P0.7 | AD7 | 32 | I2C EEPROM – SDA |
| P1 | P1.0 | T2 | 1 | LCD display – D0 |
|  | P1.1 | T2EX | 2 | LCD display – D1 |
|  | P1.2 |  | 3 | LCD display – D2 |
|  | P1.3 |  | 4 | LCD display – D3 |
|  | P1.4 |  | 5 | LCD display – E |
|  | P1.5 |  | 6 | LCD display – RS |
|  | P1.6 |  | 7 | Jazz sensor microcontroller – RESET |
|  | P1.7 |  | 8 | Power LED |
| P2 | P2.0 | A8 | 21 | Jazz sensor microcontroller – DATA0 |
|  | P2.1 | A9 | 22 | Jazz sensor microcontroller – DATA1 |
|  | P2.2 | A10 | 23 | Jazz sensor microcontroller – DATA2 |
|  | P2.3 | A11 | 24 | Jazz sensor microcontroller – DATA3 |
|  | P2.4 | A12 | 25 | Jazz sensor microcontroller – DATA0 |
|  | P2.5 | A13 | 26 | Jazz sensor microcontroller – DATA5 |
|  | P2.6 | A14 | 27 | Jazz sensor microcontroller – DATA6 |
|  | P2.7 | A15 | 28 | Jazz sensor microcontroller – DATA7 |
| P3 | P3.0 | RxD | 10 | Serial port input |
|  | P3.1 | TxD | 11 | Serial port output |
|  | P3.2 | /INT0 | 12 | Jazz-sensor microcontroller – STROBE |
|  | P3.3 | /INT1 | 13 | Voltage level monitor |
|  | P3.4 | T0 | 14 | Serial port CTS |
|  | P3.5 | T1 | 15 | Bluetooth hardware reset |
|  | P3.6 | /WR | 16 | Serial port RTS |
|  | P3.7 | /RD | 17 | Data LED |

### . Power supply and voltage level monitor

Components

- ICs

  - IC1 			LM393N comparator

  - IC2 			LM2940-CT-5.0 low-drop voltage regulator

- Capacitors

  - C1			47u/16V

  - C2			10u/10V

- Resistors

  - R1			47k

  - R2			390

  - R3			50k

- Diodes

  - D1, D2		Zener, C4V8

- Batteries

  - G1			6xAA 1,5V

- Connectors

  - CONN1		power output with voltage level monitor

  - CONN2		USB connector

### Keyboard

### PCM codec

Components

- ICs

  - IC1 			MC148493 PCM codec

  - IC2 			74HC04

- Capacitors

  - C1, C6, C7, C		100n

  - C2, C3			33p

  - C4, C5			420p

  - C8			68u

- Resistors

  - R1			10M

  - R2			100k

  - R3			2,2k

  - R4, R5, R6, R7	1k

  - R8, R9			75k

  - R10			10k

  - R11			330

  - R12			33

- Diodes

  - D1			Zener, C3V9

- Transistors

  - T1			BC237

- Connectors

  - X1			microphone jack

  - X2			earphone jack

  - CONN1		Bluetooth PCM interface

  - CONN2		Bluetooth hardware reset (input)

Connectors

| Connector | PIN number | Signal |
| --- | --- | --- |
| CONN1 Bluetooth PCM interface | 1 | +5V |
|  | 2 | PCM_CLK |
|  | 3 | WAKE_UP |
|  | 4 | PCM_SYNC |
|  | 5 | PCM_OUT |
|  | 6 | +3,3V |
|  | 7 | PCM_IN |
|  | 8 | DETACH |
|  | 9 | RESET |
|  | 10 | Ground |
| CONN2 Bluetooth hardware reset | 1 | RESET |
|  | 2 | Ground |

### Data Acquisition Unit hardware interconnection diagram

  - Appendix B. Data Acquisition Unit software modules

### Microcontroller software modules

Data Acquisition Unit microcontroller software comprises several modules. The modules are described in the table below.

| Module name | Module functions |
| --- | --- |
| main.asm | - system devices (interrupt system, Bluetooth module, Jazz sensor, serial port, LCD display, I2C bus) initialization - main program loop - system cold reset |
| serial.asm | - serial port initialization (speed, frame format) - single character sending/receiving - character string sending |
| cyclic_buffer.asm | - serial port cyclic data buffer - RTS/CTS flow control - reader/writer synchronization |
| bt_command.asm | - Bluetooth commands sending |
| bt_event.asm | - Bluetooth events parsing - Bluetooth events dispatching |
| bt_data.asm | - parsing Bluetooth data packets - command dispatching |
| bt_other.asm | - Bluetooth module initialization (reset, UART speed setting) |
| sensor.asm | - Jazz sensor initialization - Jazz sensor data fetching - Jazz sensor data transmission |
| lcd.asm | - LCD display initialization - single character display - character string display |
| button.asm | - keyboard handling routines |
| i2c.asm | - I2C bus communication (reset, I2C read/write) |
| 24c16.asm | - I2C EEPROM handling routines (byte read/write, block read/write) |
| id_card.asm | - ID card system initialization - ID card verification - ID card expiry data updating - ID card removal handling |
| pin.asm | - PIN code entering/verification |

### Microcontroller memory maps

80C52 microcontroller is equipped with 256 bytes of built-in RAM memory. In the memory program variables and data buffers are stored. The table below contains Data Acquisition Unit memory map.

| Addr | Description |  | Variable |
| --- | --- | --- | --- |
| 0xFF | ACL Data Header |  | FRAME_BUFFER |
| ... |  |  |  |
| 0xF6 |  |  |  |
| 0xF5 | Sensor Data Buffer |  | DATA_BUFFER |
| ... |  |  |  |
| 0xE1 |  |  |  |
| 0xE0 | Custom Event Text |  | CUSTOM_EVENT_TEXT |
| 0xDF |  |  |  |
| 0xDE |  |  |  |
| 0xDD |  |  |  |
| 0xDC |  |  |  |
| 0xDB |  |  |  |
| 0xDA |  |  |  |
| 0xD9 |  |  |  |
| 0xD8 |  |  |  |
| 0xD7 |  |  |  |
| 0xD6 |  |  |  |
| 0xD5 |  |  |  |
| 0xD4 |  |  |  |
| 0xD3 |  |  |  |
| 0xD2 |  |  |  |
| 0xD1 | Comand Complete Result |  | COMMAND_COMPLETE |
| 0xD0 |  |  |  |
| 0xCF |  |  |  |
| 0xCE | Unused |  |  |
| ... |  |  |  |
| 0xA0 |  |  |  |
| 0x9F | STACK |  |  |
| 0x80 |  |  |  |
| 0x7F | RS232 buffer |  |  |
| 0x40 |  |  | BUF_ADDR |
| 0x3F | RS232 buffer - reader's pointer |  | GET_POS |
| 0x3E | RS232 buffer - writer's pointer |  | PUT_POS |
| 0x3D | RS232 buffer - bytes free |  | BUF_FREE |
| 0x3C | Button state register 1 |  | BUTTON_REG_1 |
| 0x3B | Button state register 2 |  | BUTTON_REG_2 |
| 0x3A | ID card check delay |  | ID_CARD_TIMER |
| 0x39 | Bytes to send |  | BYTES_TO_SEND |
| 0x38 | Unused |  |  |
| ... |  |  |  |
| 0x30 |  |  |  |
| 0x2F | Bluetooth Address | Pin code buffer |  |
| 0x2E | Bluetooth Address | Pin code buffer |  |
| 0x2D | Bluetooth Address | Pin code buffer |  |
| 0x2C | Bluetooth Address | Pin code buffer | PIN_NUMBERS |
| 0x2B | Bluetooth Address | Pin code pointer | PIN_PTR |
| 0x2A | Bluetooth Address |  | BT_ADDR |
| 0x29 | Unused |  |  |
| 0x23 |  |  |  |
| 0x22 |  |  |  |
| 0x21 | Bit addressable memory |  |  |
| 0x20 | Bit addressable memory |  |  |
| 0x1F | Unused |  |  |
| 0x0A |  |  |  |
| 0x09 |  |  |  |
| 0x08 |  |  |  |
| 0x07 | General Purpose Registers |  |  |
| ... |  |  |  |
| 0x00 |  |  |  |

Bit addressable memory map is shown in the table below.

| Addr/bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 |  |  |  |  |  |  | AUTH | SEND NEXT BYTE |
| 0x20 | DROP FRAME | PIN VALID | TEMP | ID CARD VALID | CONN AUTH | I2C ERR | SCO CONN | ACL CONN |

  - Appendix C. Central System Unit – Core class diagram

This section contains Central System Unit core specification. The core is responsible for connection management, data flow management, data analysis, alert and database management.

### Diagram C.1 – Connection Manager Class Diagram

### Diagram C.2 – Data Flow Management Class Diagram

### Diagram C.3 – Data Analyzers Class Diagram

### Diagram C.4 – Alerts Class Diagram

### Diagram C.5 – Database Class Diagram

### Dictionary items description

Name: BT_Addr [BlueEyes.cm.]

Type: Class

Description: the structure to store BT device address

Name: CAlert [BlueEyes.alerts.]

Type: Class

Description: A class containing alarm details. It can contain simple defined alarms, custom text alarms and beeper alarms.

Name: CAlertProducer [BlueEyes.alerts.]

Type: Class

Description: A class collecting all alarms from the alarm tree and sending them through the buffer to the operator.

Name: CAlertsDatabase [BlueEyes.db.]

Type: Class

Description: A class used to store alerts tree definition

Name: CAnaliserItem [final_src.Kolejki.]

Type: Class

Description: An item on the data analysers list

Name: CAnaliserManager [final_src.Kolejki.]

Type: Class

Description: A class for managing the data analysers

Name: CAnalysisThread [final_src.Kolejki.]

Type: Class

Description: The main analysis thread

Name: CBloodAnaliser [final_src.Kolejki.]

Type: Class

Description: An analiser to provide Oxy/All value calculation

Name: CBrainAnaliser [final_src.Kolejki.]

Type: Class

Description: Eye motility analyser

Name: CBTConnection [BlueEyes.cm.]

Type: Class

Description: A class maintaining all Bluetooth communication routines using a serial port.

Name: CBufferTempl [BlueEyes.tools.]

Type: Class

Description: A template class for a FIFO buffer

Name: CComplexCondition [BlueEyes.alerts.]

Type: Class

Description: A complex condition consisting of several other conditions bound with a logic operator (AND/OR)

Name: CConditionNode [BlueEyes.alerts.]

Type: Class

Description: A general class for all kinds of conditions

Name: CConnection [BlueEyes.cm.]

Type: Class

Description: An abstract base class for connections

Name: CConnectionManager [BlueEyes.cm.]

Type: Class

Description: Connection Manager maintains the mapping between certain connections and operators in the system. It also creates and destroys operator-related objects, provides operator roaming functionality and a possibility for other classes (implementing the IOperatorStateListener interface) to register to be notified about operators' login/logout events.

Name: CDataBufferTempl [BlueEyes.tools.]

Type: Class

Description: A data buffer class template. It can contain any type of structural data. It also provides thread-safe notifying and access to the data listeners (derived from     CListenerQueue)

Name: CDataManager [final_src.Kolejki.]

Type: Class

Description: A class for managing the data sources (queues).

Name: CDataProdItem [final_src.Kolejki.]

Type: Class

Description: An item on the data source list

Name: CDAUData [BlueEyes.cm.]

Type: Class

Description: A class for storing the event to be sent to the operator (text and sound)

Name: CDBConnection [BlueEyes.cm.]

Type: Class

Description: A class emulating a real Bluetooth connection using the data stored in the database. This routine is used while the offline data playback.

Name: CExBufferTempl [BlueEyes.tools.]

Type: Class

Description: A template class for the thread-safe FIFO buffer.

Name: CFileConnection [BlueEyes.cm.]

Type: Class

Description: A class emulating a real connection using the data file.

Name: CHCIAcl [BlueEyes.cm.]

Type: Class

Description: A class used for ACL-level Bluetooth communication routines.

Name: CHCICommand [BlueEyes.cm.]

Type: Class

Description: A class used for HCI-level Bluetooth communication routines.

Name: CHemoAnaliser [final_src.Kolejki.]

Type: Class

Description: An analyser to provide pulse-based smoothing of the haemoglobin level signal.

Name: CImageStorage [BlueEyes.db.]

Type: Class

Description: A class used to store and playback audio/video data from the database.

Name: CListenerPos [BlueEyes.tools.]

Type: Class

Description: An item on the consumers list.

Name: CListenerQueue [BlueEyes.tools.]

Type: Class

Description: A class maintaining thread-safe registration/unregistration of data consumers (listeners). It implements a part of IDataProducer interface.

Name: CMiddleAnaliser [final_src.Kolejki.]

Type: Class

Description: An analyser to calculate average value of a signal.

Name: CMiddleCalc [final_src.Kolejki.]

Type: Class

Description: A class to calculate an average value.

Name: CMultiDataAnaliser [final_src.Kolejki.]

Type: Class

Description: A superclass for analysers with multiple data sources.

Name: CMyDataBase [BlueEyes.db.]

Type: Class

Description: A class covering all ODBC routines (connection to the data source, disconnecting, handle management, error responding).

Name: COperator [BlueEyes.cm.]

Type: Class

Description: A class containing all operator's data and putting all the data through the data queue.

Name: COperatorItem [BlueEyes.cm.]

Type: Class

Description: An item on the operators list.

Name: COperatorLisItem [BlueEyes.cm.]

Type: Class

Description: A single item on the Operator State Listeners list.

Name: COperDatabase [BlueEyes.db.]

Type: Class

Description: A class maintaining all operator-related data.

Name: COxyhemoAnaliser [final_src.Kolejki.]

Type: Class

Description: An analyser to provide pulse-based smoothing of the oxyhemoglobin level signal.

Name: CParallelPort [BlueEyes.Utilities.]

Type: Class

Description: A class for Windows 2000/NT parallel port access.

Name: CPieAnaliser [final_src.Kolejki.]

Type: Class

Description: An analyser to provide the data for the Pie Charts.

Name: CProcessKiller [BlueEyes.Utilities.]

Type: Class

Description: A class to provide a Windows 2000 system process management security workaround.

Name: CPulseAnaliser [final_src.Kolejki.]

Type: Class

Description: An analyser to calculate the pulse rate.

Name: CRawFrame [final_src.Kolejki.]

Type: Class

Description: A raw data frame received from the sensor.

Name: CSensorData [final_src.Kolejki.]

Type: Class

Description: A class for demultiplexing and decoding sensor data and putting it into the data queues.

Name: CSerialPort [BlueEyes.cm.]

Type: Class

Description: An overall serial port access class.

Name: CSimpleAnaliser [final_src.Kolejki.]

Type: Class

Description: A simple analyzer class.  It can work either as a self-threaded or source-threaded      analyzer.

Name: CSimpleCondition [BlueEyes.alerts.]

Type: Class

Description: A simple condition based on the raw signal data, cut level and state duration time.

Name: CStampedFrame [final_src.Kolejki.]

Type: Class

Description: A raw sensor frame with the timestamp.

Name: CSystemLogger [BlueEyes.db.]

Type: Class

Description: A class maintaining raw data logging and playback.

Name: CThread [BlueEyes.Utilities.]

Type: Class

Description: A superclass for classes with separate thread processing.

Name: CTimePercAnaliser [final_src.Kolejki.]

Type: Class

Description: An analyzer to calculate the percentage of time the value is above a certain level

Name: CTreeAnaliser [BlueEyes.Tree.]

Type: Class

Description: A decision tree analyzer

Name: CTreeClassifier [BlueEyes.Tree.]

Type: Class

Description: A simple decision tree classifier.

Name: CVarianceAnaliser [final_src.Kolejki.]

Type: Class

Description: An analyzer to calculate a variance of a signal.

Name: CVarianceCalc [final_src.Kolejki.]

Type: Class

Description: A class to calculate a variance value.

Name: CVelocityAnaliser [final_src.Kolejki.]

Type: Class

Description: An analyzer to calculate a velocity of a signal change.

Name: IDataConsumer [BlueEyes.Interfaces.]

Type: Class

Description: An abstract interface for Data Consumer class.

Name: IDataProducer [BlueEyes.Interfaces.]

Type: Class

Description: An abstract interface for data source (Data Producer).

Name: IManagableAnaliser [BlueEyes.Interfaces.]

Type: Class

Description: An abstract interface for analyzer classes that can be managed by the CAnaliserManager.

Name: IOperatorStateListener [BlueEyes.Interfaces.]

Type: Class

Description: An abstract interface for a class that wants to be notified about operators' login/logout events.

  - Appendix D. Central System Unit hardware schematics

### CSU adapter board

Components

- Transistor

  - T1			BC 237

- Diodes

  - D1, D2, D3		LED, round 3mm

- Resistors

  - R1			1k

  - R2, R3, R4		330

- Connectors

  - CONN1		PC interface

  - CONN2		Bluetooth hardware reset output

  - CONN3		Bluetooth serial interface

Connectors

| Connector | PIN number | Signal |
| --- | --- | --- |
| CONN1 PC Interface | 1 | Serial port – TxD |
|  | 2 | Serial port – CTS |
|  | 3 | Serial port – RTS |
|  | 4 | Serial port – RxD |
|  | 5 | Ground |
|  | 6 | Parallel port – D3 |
|  | 7 | Parallel port – D2 |
|  | 8 | Parallel port – D1 |
|  | 9 | Parallel port – D0 |
| CONN2 | 1 | RESET |
|  | 2 | Ground |
| CONN3 Bluetooth Serial Port | 1 | Ground |
|  | 2 | RX (input) |
|  | 3 | RTS (input) |
|  | 4 | CTS (output) |
|  | 5 | TX (output) |

### Central System Unit hardware interconnection diagram

  - Appendix E. Connection Manager specification

Connection Manager consists of 4 modules:

- Bluetooth HCI (Host Controller Interface) driver (simple stack)

- Incoming/outgoing data  management

- Serial port routines (communication with Bluetooth module)

- Parallel port routines (for additional operations i.e. Bluetooth hardware reset, display CSU state)

### Bluetooth HCI driver

We have decided to write our own simple Bluetooth communication stack in order to increase system reliability and efficiency. The Connection Manager communicates with the Bluetooth module (Bluetooth Host Controller) via standard serial port using HCI (Host Controller Interface) commands. It implements the Bluetooth HCI UART Transport Layer Protocol.

The HCI provides a uniform command method of accessing the Bluetooth hardware capabilities.  Communication with the Host Controller is based on command/event schema. HCI commands take different amounts of time to be completed. Therefore, the results of commands are reported back to the Host in the form of an event. For example, for most HCI commands the Host Controller will generate the Command Complete event when a command is completed. This event contains the return parameters for the completed HCI command. To detect errors on the HCI-Transport Layer we have defined a response timeout between the Host Controller receiving a command and sending a response to the command (e.g. a Command Complete or Command Status event).

The following diagrams show the flow control in the Bluetooth HCI Driver module.

Bluetooth module initialization

Connection establishment

Operator’s authorization

Bluetooth event and data frames dispatching

Bluetooth event parsing

HCI Communication protocol

HCI Command Packets are used to send commands from the Connection Manager to the Host Controller. Each command is assigned a 2 byte Opcode used to uniquely identify different types of commands. The Opcode parameter is divided into two fields, called the OpCode Group Field (OGF) and OpCode Command Field (OCF). The OGF occupies the upper 6 bits of the Opcode, while the OCF occupies the remaining 10 bits. Each command has a specific number of parameters associated with it. These parameters and the size of each of the parameters are defined for each command. Each parameter is an integer number of bytes in size.

The format of the HCI Command Packet is shown below.

The HCI Event Packet is used by the Host Controller to notify the Connection Manager when events occur. Each event is assigned a 1-Byte event code used to uniquely identify different types of events. Length of all of the parameters contained in this packet, measured in bytes.

The format of the HCI Event Packet is shown below.

HCI Data Packets are used to exchange data between the Host and Host Controller. The format of the HCI ACL Data Packet is shown below.

Raw data cannot be sent directly using the HCI data packet. It is therefore necessary to use a higher layer protocol – L2CAP (Logical Link Control and Adaptation Protocol). It enables applications to transmit and receive data packets up to 64 kilobytes in length. The data should be sent as the payload of the L2CAP packet. The format of L2CAP packet is shown below:

L2CAP PDU is encapsulated into the HCI Data packet. However, HCI does not provide the ability to distinguish the three HCI packet Types (i.e. HCI Command Packet, HCI Event Packet and HCI ACL Data Packet). Therefore, if the HCI packets are sent via a common physical interface (serial port) it is necessary to use HCI UART Transport Layer Protocol. All the packets must be encapsulated into the HCI UART Transport Layer packet. The format of this packet is shown below.

The HCI packet indicator (1Byte) shall be sent immediately before the HCI packet. All kinds of HCI packets have a length field, which is used to determine how many bytes are expected for the HCI packet. When an entire HCI packet has been received, the next HCI packet indicator is expected for the next HCI packet.

HCI packet indicators:

| HCI Packet Type | HCI Packet Indicator |
| --- | --- |
| HCI Command Packet | 0x01 |
| HCI ACL Data Packet | 0x02 |
| HCI Event Packet | 0x04 |

HCI commands used by the Connection Manager

- HCI_Ericsson_Set_UART_Baud_Rate

This Ericsson specific HCI command will change the UART baud rate.

Command Parameters:

UART speed: Size: 1 Byte

0x00 460800bps

0x01 230400bps

0x02 115200bps

0x03 56700bps

0x04 28800bps

0x05 14400bps

0x06 7200bps

0x07 3600bps

0x08 1800bps

0x09 900bps

Return Parameters: none

Event(s) generated:

When the HCI_Ericsson_Set_UART_Baud_Rate command has completed, a Command Complete event will be generated.

- HCI_Read_BD_Addr

This command will read the value for the BD_ADDR parameter. The BD_ADDR is a 48-bit unique identifier for a Bluetooth device.

Command Parameters: None.

Return Parameters:

Status: Size: 1 Byte

0x00 Read_BD_ADDR command succeeded.

0x01-0xFF Read_BD_ADDR command failed.

BD_ADDR: Size: 6 Bytes

0xXXXXXXXXXXXX BD_ADDR of the Device

Event(s) generated (unless masked away):

When the Read_BD_ADDR command has completed, a Command Complete event will be generated.

- HCI_Write_Scan_Enable

This command will write the value for the Scan_Enable parameter. The Scan_Enable parameter controls whether or not the Bluetooth device will periodically scan for page attempts and/or inquiry requests from other Bluetooth devices. If Page_Scan is enabled, then the device will enter page scan mode based on the value of the Page_Scan_Interval and Page_Scan_Window parameters. If Inquiry_Scan is enabled, then the device will enter Inquiry Scan mode based on the value of the Inquiry_Scan_Interval and Inquiry_Scan_ Window parameters.

Command Parameters:

Scan_Enable: Size: 1 Byte

0x00 No Scans enabled. Default.

0x01 Inquiry Scan enabled. Page Scan disabled.

0x02 Inquiry Scan disabled. Page Scan enabled.

0x03 Inquiry Scan enabled. Page Scan enabled (used by Connection Manager)

Return Parameters:

Status: Size: 1 Byte

0x00 Write_Scan_Enable command succeeded.

0x01-0xFF Write_Scan_Enable command failed.

Event(s) generated (unless masked away):

When the Write_Scan_Enable command has completed, a Command Complete event will be generated.

- HCI_Inquiry_Cancel

This command will cause the Bluetooth device to stop the current Inquiry if the Bluetooth device is in Inquiry Mode. This command allows the Host to interrupt the Bluetooth device and request the Bluetooth device to perform a different task. The command should only be issued after the Inquiry command has been issued, a Command Status event has been received for the Inquiry command, and before the Inquiry Complete event occurs.

Return Parameters:

Status: Size: 1 Byte

0x00 Inquiry_Cancel command succeeded.

0x01-0xFF Inquiry_Cancel command failed.

Event(s) generated (unless masked away):

When the Inquiry Cancel command has completed, a Command Complete event will be generated. No Inquiry Complete event will be generated for the canceled Inquiry process.

- HCI_Create_Connection

This command will cause the Link Manager to create a connection to the Bluetooth device with the BD_ADDR specified by the command parameters. This command causes the local Bluetooth device to begin the Page process to create a link level connection. The Link Manager will determine how the new ACL connection is established. The Packet_Type command parameter specifies which packet types the Link Manager shall use for the ACL connection. The Page_Scan_Repetition_Mode and Page_Scan_Mode parameters specify the page scan modes supported by the remote device with the BD_ADDR. This is the information that was acquired during the inquiry process. The Clock_Offset parameter is the difference between its own clock and the clock of the remote device with BD_ADDR. Only bits 2 through 16 of the difference are used, and they are mapped to this parameter as bits 0 through 14 respectively. A  Clock_Offset_Valid_Flag, located in bit 15 of the Clock_Offset parameter, is used to indicate if the Clock Offset is valid or not. A Connection handle for this connection is returned in the Connection Complete event (see below). The Allow_Role_Switch parameter specifies if the local device accepts or rejects the request of a master-slave role switch when the remote device requests it at the connection setup (in the Role parameter of the Accept_Connection_Request command).

Command Parameters:

BD_ADDR: Size: 6 Bytes

0xXXXXXXXXXXXX BD_ADDR of the Device to be connected.

Packet_Type: Size: 2 Bytes

0x0001 Reserved for future use.

0x0002 Reserved for future use.

0x0004 Reserved for future use.

0x0008 DM1

0x0010 DH1 (used by Connection Manager)

0x0020 Reserved for future use.

0x0040 Reserved for future use.

0x0080 Reserved for future use.

0x0100 Reserved for future use.

0x0200 Reserved for future use.

0x0400 DM3

0x0800 DH3

0x1000 Reserved for future use.

0x2000 Reserved for future use.

0x4000 DM5

0x8000 DH5

Page_Scan_Repetition_Mode: Size: 1 Byte

0x00 R0 (used by Connection Manager)

0x01 R1

0x02 R2

0x03 – 0xFF Reserved.

Page_Scan_Mode: Size: 1 Byte

0x00 Mandatory Page Scan Mode  (used by Connection Manager)

0x01 Optional Page Scan Mode I.

0x02 Optional Page Scan Mode II.

0x03 Optional Page Scan Mode III.

0x04 – 0xFF Reserved.

Clock_Offset: Size: 2 Bytes

Bit 14.0 Bit 16.2 of CLKslave-CLKmaster.

Bit 15 Clock_Offset_Valid_Flag

Invalid Clock Offfset = 0

Valid Clock Offset = 1  (used by Connection Manager)

Allow_Role_Switch: Size: 1 Byte

0x00 The local device will be a master, and will not accept a master-slave

switch requested by the remote device at the connection setup. (used by 
Connection Manager)

0x01 The local device may be a master, or may become a slave after

accepting a master-slave switch requested by the remote device at the

connection setup.

0x02-0xFF Reserved for future use.

Return Parameters:

None.

Event(s) generated (unless masked away):

When the Host Controller receives the Create Connection command, the Host Controller sends the Command Status event to the Host. In addition, when the LM determines the connection is established, the Host Controller, on both Bluetooth devices that form the connection, will send a Connection Complete event to each Host. The Connection Complete event contains the Connection Handle if this command is successful.

- HCI_PIN_Code_Request_Reply

The PIN_Code_Request_Reply command is used to reply to a PIN Code request event from the Host Controller, and specifies the PIN code to use for a connection. The PIN Code Request event will be generated when a connection with remote initiating device has requested pairing.

Command Parameters:

BD_ADDR: Size: 6 Bytes

0xXXXXXXXXXXXX  - BD_ADDR of the Device which the PIN code is for.

PIN_Code_Length: Size: 1 Byte

0xXX The PIN code length specifics the length, in bytes, of the PIN code to be used.

Range: 0x01-0x10. (Connection Manager uses 8 bytes long PIN codes)

PIN_Code: Size: 16 Bytes

0xXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

PIN code for the device that is to be connected. The Host should insure

that strong PIN Codes are used. PIN Codes can be up to a maximum of

128 bits. The MSB of the PIN Code occupies byte zero.

Return Parameters:

Status: Size: 1 Byte

0x00 PIN_Code_Request_Reply command succeeded.

0x01-0xFF PIN_Code_Request_Reply command failed.

BD_ADDR: Size: 6 Bytes

0xXXXXXXXXXXXX  BD_ADDR of the Device which the PIN Code request reply  
has completed.

Event(s) generated (unless masked away):

The PIN_Code_Request_Reply command will cause a Command Complete event to be generated.

- HCI_Disconnect

The Disconnection command is used to terminate an existing connection. The Connection_Handle command parameter indicates which connection is to be disconnected. The Reason command parameter indicates the reason for ending the connection. The remote Bluetooth device will receive the Reason command parameter in the Disconnection Complete event. All SCO connections on a physical link are disconnected before the ACL connection on the same physical connection is disconnected.

Command Parameters:

Connection_Handle: Size: 2 Bytes (12 Bits meaningful)

0xXXXX Connection Handle for the connection being disconnected.

Range: 0x0000-0x0EFF (0x0F00 - 0x0FFF Reserved for future use)

Reason: Size: 1 Byte

0x13-0x15, 0x1A -  Other End Terminated Connection error codes (0x13-0x15) and Unsupported Remote Feature error code (0x1A).

Return Parameters: None.

Event(s) generated (unless masked away):

- HCI_Write_Encryption_Mode

This command will write the value for the Encryption_Mode parameter. The Encryption_Mode parameter controls if the local device requires encryption to the remote device at connection setup. At connection setup, only the device(s) with the Authentication_Enable parameter enabled and Encryption_Mode parameter enabled will try to encrypt the connection to the other device.

Command Parameters:

Encryption_Mode: Size: 1 Byte

0x00 Encryption disabled. Default.

0x01 Encryption only for point-to-point packets. (used by Connection Manager)

0x02 Encryption for both point-to-point and broadcast packets.

0x03-0xFF Reserved.

Return Parameters:

Status: Size: 1 Byte

0x00 Write_Encryption_Mode command succeeded.

0x01-0xFF Write_Encryption_Mode command failed.

Event(s) generated (unless masked away):

When the Write_Encryption_Mode command has completed, a Command Complete event will be generated.

- HCI_Add_SCO_Connection

This command will cause the link manager to create a SCO connection using the ACL connection specified by the Connection_Handle command parameter. This command causes the local Bluetooth device to create a SCO connection. The Link Manager will determine how the new connection is established. This connection is determined by the current state of the device, its piconet, and the state of the device to be connected. The Packet_Type command parameter specifies which packet types the Link Manager should use for the connection. SCO-Connection can only be created when an ACL Connection already exists.

Command Parameters:

Connection_Handle Size 2 Bytes (12 Bits meaningful)

0xXXXX Connection Handle for the ACL connection being used to create an SCO connection. Range: 0x0000-0x0EFF (0x0F00 - 0x0FFF Reserved for future use)

Packet_Type: Size: 2 Bytes

0x0001 Reserved for future use.

0x0002 Reserved for future use.

0x0004 Reserved for future use.

0x0008 Reserved for future use.

0x0010 Reserved for future use.

0x0020 HV1 (used by Connection Manager)

0x0040 HV2

0x0080 HV3

0x0100 Reserved for future use.

0x0200 Reserved for future use.

0x0400 Reserved for future use.

0x0800 Reserved for future use.

0x1000 Reserved for future use.

0x2000 Reserved for future use.

0x4000 Reserved for future use.

0x8000 Reserved for future use.

Return Parameters: None.

Event(s) generated (unless masked away):

When the Host Controller receives the Add_SCO_Connection command, it sends the Command Status event to the Connection Manager. In addition, when the LM determines the connection is established, the Host Controller, on both Bluetooth devices that form the connection, will send a Connection Complete event to each Host. The Connection Complete event contains the Connection Handle if this command is successful.

Complete event will indicate that this command has been completed.

HCI commands used by the Connection Manager

- HCI_Command_Complete_Event

The Command Complete event is used by the Host Controller for most commands to transmit return status of a command and the other event parameters that are specified for the issued HCI command. The Num_HCI_Command_Packets event parameter allows the Host Controller to indicate the number of HCI command packets the Host can send to the Host Controller. If the Host Controller requires the Host to stop sending commands, the Num_HCI_Command_Packets event parameter will be set to zero. To indicate to the Host that the Host Controller is ready to receive HCI command packets, the Host Controller generates a Command Complete event with the Command_Opcode 0x0000, and the Num_HCI_Command_Packets event parameter is set to 1 or more. Command_Opcode, 0x0000 is a NOP (No OPeration), and can be used to change the number of outstanding HCI command packets that the Host can send before waiting. See each command for the parameters that are returned by this event.

Event Parameters:

Num_HCI_Command_Packets: Size: 1 Byte

N = 0xXX The Number of HCI command packets which are allowed to be sent to the Host Controller from the Host. Range for N: 0 – 255

Command_Opcode: Size: 2 Bytes

0xXXXX Opcode of the command which caused this event.

Return_Parameter(s): Size: Depends on Command

0xXX This is the return parameter(s) for the command specified in the Command_Opcode event parameter.

- HCI_Inquiry_Result_Event

The Inquiry Result event indicates that a Bluetooth device or multiple Bluetooth devices have responded so far during the current Inquiry process. This event will be sent from the Host Controller to the Connection Manager as soon as an Inquiry Response from a remote device is received if the remote device supports only mandatory paging scheme. The Host Controller may queue these Inquiry Responses and send multiple Bluetooth devices information in one Inquiry Result event. The event can be used to return one or more Inquiry responses in one event. This event contains the BD_ADDR, Page_Scan_Repetition_Mode, Page_Scan_Period_Mode, Page_Scan_Mode, Clock_Offset, and the Class of Device for each Bluetooth device that responded to the latest inquiry.

Event Parameters:

Num_Responses: Size: 1 Byte

0xXX Number of responses from the Inquiry.

BD_ADDR[i]: Size: 6 Bytes * Num_Responses

0xXXXXXXXXXXXX BD_ADDR for each device which responded.

Page_Scan_Repetition_Mode[i]: Size: 1 Byte* Num_Responses

0x00 R0

0x01 R1

0x02 R2

0x03 – 0xFF Reserved

Page_Scan_Period_Mode[i]: Size: 1 Byte* Num_Responses

0x00 P0

0x01 P1

0x02 P2

0x03 – 0xFF Reserved

Page_Scan_Mode[i]: Size: 1 Byte* Num_Responses

0x00 Mandatory Page Scan Mode

0x01 Optional Page Scan Mode I

0x02 Optional Page Scan Mode II

0x03 Optional Page Scan Mode III

0x04 – 0xFF Reserved

Class_of_Device[i]: Size: 3 Bytes * Num_Responses

0xXXXXXX Class of Device for the device

Clock_Offset[i]: Size: 2 Bytes * Num_Responses

Bit 14.0 Bit 16.2 of CLKslave-CLKmaster.

Bit 15 Reserved

- HCI_Inquiry_Complete_Event

The Inquiry Complete event indicates that the Inquiry is finished. This event contains a status parameter, which is used to indicate if the Inquiry completed successfully or if the Inquiry was not completed. In addition, the Num_Responses parameter contains the number of Bluetooth devices, which responded during the latest inquiry.

Event Parameters:

Status: Size: 1 Byte

0x00 Inquiry command completed successfully.

0x01-0xFF Inquiry command failed.

Num_Responses: Size: 1 Byte

0xXX Number of responses from the Inquiry.

- HCI_Connection_Complete_Event

The Connection Complete event indicates to both of the Hosts forming the connection that a new connection has been established. This event also indicates to the Host, which issued the Create Connection, Add_SCO_Connection, or Accept_Connection_Request or Reject_Connection_Request command and then received a Command Status event, if the issued command failed or was successful.

Event Parameters

Status: Size: 1 Byte

0x00 Connection successfully completed.

0x01-0xFF Connection failed to Complete.

Connection_Handle: Size: 2 Bytes (12 Bits meaningful)

0xXXXX Connection Handle to be used to identify a connection between to Bluetooth devices. The Connection Handle is used as an identifier for transmitting and receiving voice or data.  Range: 0x0000-0x0EFF (0x0F00 - 0x0FFF Reserved for future use)

BD_ADDR: Size: 6 Bytes

0xXXXXXXXXXXXX BD_ADDR of the other connected Device forming the connection.

Link_Type: Size: 1 Byte

0x00 SCO connection (Voice Channels).

0x01 ACL connection (Data Channels).

0x02-0xFF Reserved for Future Use.

Encryption_Mode: Size: 1 Byte

0x00 Encryption disabled.

0x01 Encryption only for point-to-point packets.

0x02 Encryption for both point-to-point and broadcast packets.

0x03-0xFF Reserved.

- HCI_Pin_Code_Request_Event

The PIN Code Request event is used to indicate that a PIN code is required to create a new link key. When the Host Controller generates a PIN Code Request event in order for the local Link Manager to respond to the request from the remote Link Manager, the Connection Manager responds with a PIN_Code_Request_Reply.

Event Parameters:

BD_ADDR: Size: 6 Bytes

0xXXXXXXXXXXXX BD_ADDR of the Device which a new link key is being created for.

- HCI_Link_Key_Notification_Event

The Link Key Notification event is used to indicate to the Host that a new Link Key has been created for the connection with the device specified in BD_ADDR.

Event Parameters:

BD_ADDR: Size: 6 Bytes

0xXXXXXXXXXXXX BD_ADDR of the Device for which the new link key has been

generated.

Link_Key: Size: 16 Bytes

0xXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX Link Key for the associated BD_ADDR.

- HCI_Disconnection_Complete_Event

The Disconnection Complete event occurs when a connection is terminated. The status parameter indicates if the disconnection was successful or not. The reason parameter indicates the reason for the disconnection if the disconnection was successful.

Event Parameters:

Status: Size: 1 Byte

0x00 Disconnection has occurred.

0x01-0xFF Disconnection failed to complete.

Connection_Handle: Size: 2 Bytes (12 Bits meaningful)

0xXXXX Connection Handle which was disconnected.

Range: 0x0000-0x0EFF (0x0F00 - 0x0FFF Reserved for future use)

Reason: Size: 1 Byte

0x08, 0x13-0x16, 0x1A - Connection Timeout (0x08), Other End Terminated Connection error codes (0x13-0x15), Connection Terminated by Local Host (0x16), and Unsupported Remote Feature error code (0x1A).

- Hardware_Error_Event

The Hardware Error event is used to indicate some type of hardware failure for the Bluetooth device. This event is used to notify the Host that a hardware failure has occurred in the Bluetooth module.

Event Parameters:

Hardware_Code: Size: 1 Byte

0x00 These Hardware_Codes will be implementation-specific

- Buffer_Overflow_Event

This event is used to indicate that the Host Controller’s data buffers have been overflowed. The Link_Type parameter is used to indicate that the overflow was caused by ACL data.

Event Parameters:

Link_Type: Size: 1 Byte

0x00 SCO Buffer Overflow (Voice Channels).

0x01 ACL Buffer Overflow (Data Channels).

0x02-0xFF Reserved for Future Use.

### Incoming/outgoing data  management

All packets are encapsulated into the L2CAP packets, which are in turn put into the HCI ACL Data Packets.

Packet sent from the CSU to the DAU

The packet contains the control information and the events that were detected by the analysis module.

| 1B | 0B/8B |
| --- | --- |
| Header | Payload |

Header:

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Function | data transfer from Jazz sensor 1 – start 0 – stop | Alarm buzzer control 0 – off 1 – on | force Jazz sensor calibration | Event ID 0 – no event (command only) 1 – custom event. Text message (8 characters) is required in the payload. 2-32 – built-in event. Text message is stored in the internal memory of the DAU (no payload). |  |  |  |  |

The payload contains an 8-character long custom event text. The payload should be sent only when the event Id is equal to 1. Otherwise there must be no payload.

Packets sent from DAU to CSU:

| 1B | 2B / 792B |
| --- | --- |
| Header | Payload |

Header is a packet type indicator:

0x80 – operator ID packet

0x00 – physiological data packet

Payload:

- operator ID packet

The payload contains the operator ID.

(LSB) 2 bytes (MSB)

Operator’s ID

- physiological data packet

The payload (792 bytes) contains 36 Jazz sensor frames (22 Bytes/176 bits each).

The Seq.no field (sequential number) contains the number of frames received by the DAU microcontroller from the Jazz sensor.

### Serial port routines

We have used the standard Windows API functions to communicate with the Bluetooth Host Controller over the serial port. The functions are listed below.

- CreateFile - opens the serial port

- GetCommState – gets the current control settings for the serial port

- SetCommState – sets the serial port parameters, changes the baud rate

- GetCommTimeouts – reads the time-out settings for the serial port

- SetCommTimeouts  - sets the time-out parameters

- PurgeComm – discards the data waiting in the serial port buffer

- ReadFile – reads the data from the serial port

- WriteFile – sends the data over the serial port

The HCI UART Transport Layer uses the following settings for RS232:

- RTS/CTS flow control to prevent UART buffer overrun

- Baud rate: 57600 bps after power-up or reset, 115200 bps during the normal system operation

- Number of data bits: 8

- Parity bit: no parity

- Stop bit: 1 stop bit

### Parallel port routines

Additional input/output lines were needed in order to perform Bluetooth hardware reset and to display the information about the Connection Manager state. We have decided to use the TTL-compliant PC parallel port lines.

The routines read the parallel port data register, change the appropriate bits and write the result to the port register.

Bit #0 is used to perform the Bluetooth hardware reset. Writing “1” forces the Bluetooth device to enter the reset mode. After a short delay (100 ms) bit #0 is cleared letting the Bluetooth module enter the normal operating mode. To confirm the reset operation the Bluetooth module sends two characters (0x30 ‘0’, 0x30 ‘0’) over the serial port at 57600 bps.

Bits 1-3 are used to display the Connection Manager state. These lines are connected to the LED diodes (see Appendix D).

Writing “1” (“0”) turns on (off) appropriate LED diode.

Diode 1 (bit #1) – turned on when the serial port is used.

Diode 2 (bit #4) – turned on when the Bluetooth module is in the Inquiry state

Diode 3 (bit #3) – turned on when a connection is established

  - Appendix F. C4.5 algorithm

Decision tree induction algorithm is an example-based algorithm. The input for the algorithm consists of a number of examples (each example is a vector of attributes describing a situation and a correct answer (decision class) that the module should generate).

The main idea of the algorithm is to recursively find the best attribute and its value that splits the example set into two simplest subsets. The main measure of set’s simplicity is entropy. Tree leaves are point decision class.

The algorithm works as follows:

- Create tree root. Make it current tree node.

- Check if current set consist of elements that have the same decision class. If so, current node becomes a leaf with decision class the same as in examples.

- If not: for each attribute i:

  - Sort the set by i-th attribute (q-sort implemented)

  - Calculate entropy of the whole set.

  - For each division of the sorted set into two subsets calculate:

    - Entropy of each of two subsets.

    - Estimate entropy of division ie. (size of each subset * its entropy) / (size of all subsets). The difference between initial set and division entropy and describes amount of entropy gained.

    - Calculate preference factor to favor balanced divisions given by formula:

A⋅lnAS+B⋅lnBS−S,  where

S denotes the size of the initial set,

A, B denote the sizes of the subsets.

    - Multiply entropy gained by preference factor

  - Choose division with highest value in point (iv).

- Choose the attribute with value in point (d) and update current node. Repeat points 2 – 4 for each subset taking as a new current node left and right child of current node respectively.

  - Appendix G. Database schema

### Alert conditions and actions tables

### Operator personal and physiological data tables

### Detailed table description

Table: 	ADMINS

| Field | Type | Size |
| --- | --- | --- |
| Login | varchar | (20) |
| Password | varchar | (20) |
| Privs | int |  |

Table:   BlueTooth

| Field | Type | Size |
| --- | --- | --- |
| BT_ID | int |  |
| BD_ADDR | binary | (6) |
| BT_PIN | binary | (8) |

Table:	CameraImage

| Field | Type | Size |
| --- | --- | --- |
| TIMESTAMP | int |  |
| CAMERA_ID | smallint |  |
| DATALEN | int |  |
| DATA | image |  |

Table:	Groups

| Field | Type | Size |
| --- | --- | --- |
| ID | int |  |
| Name | varchar | 20 |

Table:	Position

| Field | Type | Size |
| --- | --- | --- |
| ID | int |  |
| Name | varchar | (30) |

Table:	AlertSaves

| Field | Type | Size |
| --- | --- | --- |
| TimeStamp | numeric | (10, 0) |
| AdminLogin | varchar | (20) |

Table:	Operator

| Field | Type | Size |
| --- | --- | --- |
| ID | int |  |
| Name | varchar | (20) |
| Surname | varchar | (40) |
| PicLen | int |  |
| Picture | image |  |
| Pos_ID | int |  |
| Group_ID | int |  |

Table:	Alerts

| Field | Type | Size |
| --- | --- | --- |
| TimeStamp | numeric | (10, 0) |
| Name | varchar | (16) |
| Beeper | tinyint |  |
| Text | varchar | (16) |

Table:	Log

| Field | Type | Size |
| --- | --- | --- |
| OPER_ID | int |  |
| TIMESTAMP | int |  |
| OPERATION | tinyint |  |

Table:	SystemData

| Field | Type | Size |
| --- | --- | --- |
| Oper_ID | int |  |
| TimeStamp | int |  |
| NoPacket | int |  |
| Data | binary | (6600) |

Table:	Uses_BT

| Field | Type | Size |
| --- | --- | --- |
| Oper_ID | int |  |
| BT_ID | int |  |

Table:	AlertItem

| Field | Type | Size |
| --- | --- | --- |
| TimeStamp | numeric | (10, 0) |
| ID | int |  |
| NodeName | varchar | (16) |
| AlertName | varchar | (16) |
| Type | tinyint |  |

Table:	ComplexAlertItem

| Field | Type | Size |
| --- | --- | --- |
| TimeStamp | numeric | (10, 0) |
| ID | int |  |
| Type | tinyint |  |

Table:	SimpleAlertItem

| Field | Type | Size |
| --- | --- | --- |
| TimeStamp | numeric | (10, 0) |
| ID | int |  |
| Signal_Name | varchar | (30) |
| CutLevel | int |  |
| Relation | tinyint |  |
| Duration | int |  |

Table:	Uses

| Field | Type | Size |
| --- | --- | --- |
| TimeStamp | numeric | (10, 0) |
| ID | int |  |
| USE_ID | int |  |

  - Appendix H. Camera UDP protocol

CLIENT

SERVER

capture

10:15:54 1.07.2001

request=0

request package

snapshot time 10:15:54 1.07.2001

response package

Last image request – sequence diagram

CLIENT

SERVER

request = 3

request package

last snapshot

snapshot  #1

10.31.16 1.07.2001

snapshot  #2

10.31.17 1.07.2001

snapshot  #3

10.31.18 1.07.2001

t

t

t

response package

response package

response package

snapshot  time

10.31.16 1.07.2001

snapshot  time

10.31.17 1.07.2001

snapshot  time

10.31.18 1.07.2001

Image series request – sequence diagram

### Package details

- request package

| 1B | 16B | 2B |
| --- | --- | --- |
| Request | Address | Port |

request 	– type of request

request field = 0 – last camera-grabbed frame request

request field > 0 – register the client for a sequence of next camera-grabbed frames (the request field determines the number of frames)

address 	– null terminated string containing client IPv4 address formatted as “150.254.33.142”

port 	– client IPv4 UDP port number

- response package

| 4B | 4B | 8000B |
| --- | --- | --- |
| Snapshot time | Real Size | Buffer |

snapshot time – local time on the server when the frame was grabbed

real size – the real size of the payload the buffer contains (max 8000 bytes)

buffer 	– the payload containing camera-grabbed frame compressed using JPEG algorithm

  - Appendix I. Visualization Module specification

### Visualization module structure

The graphical user interface is based on MDI scheme. The information is visualized using a set of GUI controls developed in the course of the project:

- graphical time series control – shows time series of a number of parameters (raw sensor data, conscious brain involvement level, eye acceleration, blood oxygenation etc.) in real time

- custom control – associated with custom data analyzers – visualizes the decision tree analysis results (e.g. operator position, gesture etc.)

- pie-chart control – shows conscious brain involvement level as a percentage value

- VU-meter control – shows parameter value (pulse, conscious brain involvement, blood oxygenation etc.) as a LED-diodes-like meter

### Sample user interface screenshots

Physiological condition visualization

Multiple operator support

Alert conditions and actions

  - Appendix J. Generated decision trees

This single decision tree determines whether the operator shook his head trying to say “Yes”, nodded his head to say “No” or just moved his head (no answer).

  - Appendix K. ID card system specification

### Working environment

Universal schema - units distribution supported using TCP/IP connection

Simplified schema - no units distribution available

### ID card hardware

ID card is implemented as a 2048 byte EEPROM – ST24C16. Communication with the device is carried on using an I2C bus. The connection scheme is shown in the figure below. Components

- IC1 – 24c16 I2C EEPROM

- CONN1 – ID-card interface connector

### ID card memory map

| Addr | Description | Value |
| --- | --- | --- |
| 0x00 | Signature | 0x42 |
| 0x01 |  | 0x6C |
| 0x02 |  | 0x75 |
| 0x03 |  | 0x65 |
| 0x04 |  | 0x45 |
| 0x05 |  | 0x79 |
| 0x06 |  | 0x65 |
| 0x07 |  | 0x73 |
| 0x08 | BT_PIN |  |
| ... |  |  |
| 0x0F |  |  |
| 0x10 | USER_ID | LSB |
| 0x11 |  | MSB |
| 0x12 | USER_PIN |  |
| 0x13 |  |  |
| 0x14 |  |  |
| 0x15 |  |  |
| 0x16 | CARD_VALID |  |

### ID card programmer

Items:

- ICs

  - IC1 		 	Atmel 89c2051 microcontroller

  - IC2 		 	MAX232

- Capacitors

  - C1, C2			33p

  - C3, C4, C5, C6	10u/16V

  - C7			1u/10V

- Resistors

  - R1, R2, R3, R4, R5	330

- Crystal

  - Q1			11,0592 MHz

- Diodes

  - D1			LED, round 3mm, yellow (busy)

  - D2			LED, round 3mm, red (error)

  - D3			LED, round 3mm, green (power)

- Connectors

  - CONN1		PC interface

  - CONN2		ID-card interface

Connectors:

| Connector | PIN number | Signal |
| --- | --- | --- |
| CONN1 PC Interface | 1 | GND |
|  | 2 | Serial port – RxD |
|  | 3 | Serial port – TxD |
|  | 4 | +5V |
| CONN2 I2C EEPROM ID-card | 1 | Ground |
|  | 2 | SCL |
|  | 3 | SDA |
|  | 4 | +5V |

### ID card programmer serial port protocol

ID card programmer communicates with the PC computer using standard RS232 serial port.

The programmer uses the following RS232 settings:

- Baud rate: 57600 bps

- Number of data bits: 8

- Parity bit: no parity

- Stop bit: 1 stop bit

- Flow control:  no flow control

We have designed and implemented a simple communication protocol, which enables the PC to read and write the EEPROM memory data.

The communication is based on request/response schema. Requests are sent by the PC. The programmer is passive – it can only respond to the PC requests. Requests and responses have the same packet format.

We have designed two types of the communication packets: a Control Packet and a Data Packet.

Control Packet

The Control Packet describes the operation preformed on the EEPROM memory.

| 1B | 1B | 1B | 0-258B |
| --- | --- | --- | --- |
| Header | Command | Size | Payload |

Header: request/response indicator

- 0x3C (‘<’) – response message (from programmer)

- 0x3E (‘>’) – request message (from PC)

Command:

- 0x52 (‘R’) – read operation

- 0x57 (‘W’) – write operation

- 0x45 (‘E’) – error message (response)

The Size field determines the size of the data in the Payload field. If the payload size is greater than 254 bytes (possible only if a large Data Packet is sent in the payload), the Size is equal to 255 and packet size is calculated using the Data size field of the Data Packet.

Data Packet

The Data Packet contains the data read from (wrote to) the ID card and additional information about the bank number and the address within the bank. This packet is encapsulated into the Control Packet as the payload.

Data is transmitted if the message is a read response or a write request. Otherwise the Data field is empty.

| 1B | 1B | 1B | 0-255B |
| --- | --- | --- | --- |
| Bank | Address | Size | Data |

Bank: 		EEPROM bank number

Address:	Address within the bank

Size:		Length of the Data field when the data is transmitted (read response or write request). Otherwise it contains the number of bytes of the data to be read from the memory (read request) or the number of bytes wrote to the memory (write response)

Examples:

- Read request – read 2 bytes from bank 1 at address 4 (the Data Packet with  empty data filed should be sent)

| Header | Command | Size | Payload – Data packet |  |  |
| --- | --- | --- | --- | --- | --- |
| ‘>’ | ‘R’ | 3 | Bank | Address | Size |
|  |  |  | 1 | 4 | 2 |

Write response is analogous (Header contains ‘<’).

- Write request – write 4 bytes (0x10, 0x20, 0x30, 0x40) to the bank 2 at address 1.

| Header | Command | Size | Payload – “data packet” |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘>’ | ‘W’ | 7 | Bank | Address | Size | Data |  |  |  |
|  |  |  | 2 | 1 | 4 | 0x10 | 0x20 | 0x30 | 0x40 |

Read response is analogous (Header contains ‘<’).

- Error response – sent from the programmer when an error occurs. (Control Packet contains no payload)

| Header | Command | Size |
| --- | --- | --- |
| ‘<’ | ‘E’ | 0 |

### ID card programmer TCP/IP network protocol

ID card reading sequence diagram

ID card writing sequence diagram

Package details

- simple request package

1B

Request

Request = ‘W’ 	– writing to ID card

Request = ‘R’		– reading from ID card

- universal package

| 16B | 2B | 16B | 1B | 1B | 4B |
| --- | --- | --- | --- | --- | --- |
| SystemPIN | UserID | UserPin | nTries | bResult | nError |

SystemPIN 	– an array of characters determining user’s System PIN code

UserID 	– user identifier

UserPIN 	– an array of characters determining User PIN code

nTries 	– used for various types of ID card usage limitation

bResult 	– result of performed action (read or write)

nError 	– error code if bResult is FALSE

  - Appendix L. Source code statistics

The table below shows the system complexity (CSU part) according to McCabe’s complexity measure.

| Condition statement | Occurrences |
| --- | --- |
| if | 1747 |
| and | 321 |
| or | 51 |
| case | 153 |
| while | 119 |
| for | 452 |
| TOTAL | 2843 |

Source code size:

| System part | Programming language | KLOC |
| --- | --- | --- |
| CSU | Microsoft Visual C++ | 33 |
| DAU | 8051-family assembler | 4 |

Estimated time complexity including software and hardware design, implementation and testing is 3000 person hours.

# Contents

1. Introduction	4

2. System overview	5

Performance requirements	6

Design methodologies	7

Innovative ideas	7

3. Implementation and engineering considerations	8

3.1. Functional design	8

3.2. Data Acquisition Unit (DAU)	11

3.2.1. Physiological data sensor	11

3.2.2. Hardware specification	12

3.2.3. Microcontroller software specification	13

3.2.4. DAU Project and implementation tradeoffs and optimizations	16

3.2.5. DAU Validation and testing	18

3.3. Central System Unit (CSU)	19

3.3.1. Connection Manager	19

3.3.2. Data Analysis Module	21

3.3.3. Data Logger Module	26

3.3.4. Visualization Module	27

3.3.5. CSU Project and implementation tradeoffs and optimizations	27

3.3.6. CSU Validation and testing	28

3.4. Tools developed during the course of the project	29

4. Summary	31

5. References	32

6. Appendices	33

6.1. Appendix A. Data Acquisition Unit hardware schematics	33

6.1.1. Microcontroller board	33

6.1.2. Power supply and voltage level monitor	37

6.1.3. Keyboard	38

6.1.4. PCM codec	39

6.1.5. Data Acquisition Unit hardware interconnection diagram	41

6.2. Appendix B. Data Acquisition Unit software modules	42

6.2.1. Microcontroller software modules	42

6.2.2. Microcontroller memory maps	43

6.3. Appendix C. Central System Unit – Core class diagram	45

6.3.1. Diagram C.1 – Connection Manager Class Diagram	46

6.3.2. Diagram C.2 – Data Flow Management Class Diagram	47

6.3.3. Diagram C.3 – Data Analyzers Class Diagram	48

6.3.4. Diagram C.4 – Alerts Class Diagram	49

6.3.5. Diagram C.5 – Database Class Diagram	49

6.3.6. Dictionary items description	50

6.4. Appendix D. Central System Unit hardware schematics	57

6.4.1. CSU adapter board	57

6.4.2. Central System Unit hardware interconnection diagram	58

6.5. Appendix E. Connection Manager specification	59

6.5.1. Bluetooth HCI driver	59

6.5.2. Incoming/outgoing data  management	81

6.5.3. Serial port routines	83

6.5.4. Parallel port routines	83

6.6. Appendix F. C4.5 algorithm	85

6.7. Appendix G. Database schema	86

6.7.1. Alert conditions and actions tables	86

6.7.2. Operator personal and physiological data tables	87

6.7.3. Detailed table description	88

6.8. Appendix H. Camera UDP protocol	91

6.8.1. Last image request – sequence diagram	91

6.8.2. Image series request – sequence diagram	91

6.8.3. Package details	92

6.9. Appendix I. Visualization Module specification	93

6.9.1. Visualization module structure	93

6.9.2. Sample user interface screenshots	94

6.10. Appendix J. Generated decision trees	96

6.11. Appendix K. ID card system specification	97

6.11.1. Working environment	97

6.11.2. ID card hardware	97

6.11.3. ID card memory map	98

6.11.4. ID card programmer	99

6.11.5. ID card programmer serial port protocol	100

6.11.6. ID card programmer TCP/IP network protocol	103

6.12. Appendix L. Source code statistics	105

7. Contents	106

Saccades are rapid eye jumps to new locations within a visual environment assigned predominantly by the conscious attention process.

While compressing, if the result of JPEG algorithm is larger than 8000 bytes, the compression is repeated using a stronger compression factor (worse quality, but smaller result)

Grabbed frames dimensions are 320 x 240 pixels

T. J. McCabe, "A Complexity Measure," IEEE Transactions on Software Engineering, vol. SE-2, Dec. 1976. 20